DECOCHOC H2O

# DECOCHOC H2O

PROTECTIVE AND COVERING PANEL FOR WET AREAS

Description: protective and covering panels for wet areas (such as SPM Decochoc H2O) made of rigid antibacterial PVC, class M1 (Bs2d0), and through-coloured. Its

surface is slightly textured with a fine and matte grain finish (such as SPM Hydr’X grain) perfectly suitable for wet areas due to its water-repellent qualities that limit the development of limestone while ensuring optimal scratch resistance. It is 2.50 m long, 1.30 m wide and 1.5 mm thick.

To ensure watertightness of the system, the panels are heat-sealed to each other and to the PVC coving by means of a welding rod (such as SPM H2O welding rod). Any connections with other materials are made with a silicone joint (such as SPM H2O silicone joint). The behaviour of the system in wet areas has been validated through an ATEx certification issued by the French institute CSTB.

The chemical and stain resistance of the panel must have been proven according to ISO 26987 on common cleaning products.

No heavy metals and no substance potentially subjected to any REACH restriction are used in its manufacture nor PBT or BPA. The calcium-zinc thermal stabilisation process is used. The emission level of volatile substance in inside air has been tested according to ISO 16000 and is under 15μg/m3.100% of the product are recyclable.

Colour: selected by architects from manufacturer’s standard range.

Installation sequence: glue as per manufacturer’s instructions.