CONTACT

# CONTACT

350 or 200 height protection plates

Description: adhesive protection plates (Contact 350 or 200 as manufactured by Gerflor) comprising smooth and antibacterial PVC profiles with a depth of 350 or 200 mm, 2.5 mm thick.

PVC profiles achieve Bs2d0 fire rating, feature solid colour and a smooth surface finish with decorative grooves.

A protective film is specified to minimise cleaning before acceptance.

Sections are delivered with 2 or 3 strips of adhesive foam for fixing.

No heavy metals and no substance potentially subjected to any REACH restriction are used in its manufacture. The calcium-zinc thermal stabilisation process is used. The emission level of volatile substance in inside air has been tested according to ISO 16000 and is very low (A+) according to the French regulation (23 march 2011 nr 2011-321 Decree and 19 April 2011 Order). 100% of the product are recyclable.

Colour: selected by architects from manufacturer’s standard range.

Installation sequence: adhesive. Gerflor universal mastic-glue is recommended for stronger fixing..