IMPACT

# IMPACT

200/120 or 60 height wall protection rails

Description: curved wall protection system made of smooth PVC (Impact 200/120 or 60 as manufactured by Gerflor). PVC rails project 30 mm, are 200/120 or 60 mm deep and snap-fix to aluminium brackets or on 2 continuous aluminium cores (optional). 3 mm thick, Bs2d0 fire-rated and solid colour, the PVC rails provide a smooth surface finish with decorative grooves. A protective film is specified to minimise cleaning before acceptance. Finish is provided by end pieces, external and internal 90° smooth PVC angle pieces and aluminium joint blocks.

No heavy metals are used in its manufacture and the calcium-zinc thermal stabilisation process is used. The emission level of volatile substance in inside air has been tested according to ISO 16000 and is very low (A+) according to the french regulation (23 march 2011 nr 2011-321 Decree and 19 April 2011 Order). 100% of the product are recyclable.

Colour: selected by architects from manufacturer’s standard range.

Installation sequence: recommended height of protection rail centerline 0.80 m above floor level for mid-level wall protection or above skirting for low level wall protection. Snap-fixes to aluminium brackets spaced at 0.75 m centres or continuous aluminium cores (optional).